| Overview: | Goals: |  |
| --- | --- | --- |
| Students will write a Python program on their calculator to create sounds using the built-in speaker on the TI-Innovator Hub. They will also experiment with changing the length of time in which each sound plays. Once they have some experience, they will write a program to play a song. |  | Students will: - learn to create, edit and run programs from the editor. - write a Python program that controls the built-in speaker on the TI-Innovator Hub. - write a program that controls how long a sound will play. Note – A good foundation for this project can be found at https://education.ti.com/en/activities/ti-codes/python/ti-84p-ce-python Go to Unit 1: Getting Started With Python and the TI-Innovator™ Hub and then “Skill Builder 3 – Input and Sound”. Also, Unit 2: For Loops with TI-Innovator™ Hub and then “Skill Builder 3 - Looping with Sound” has some related concepts that may be helpful. A summary of some basic Music Theory can be found on the last page of this document. |  |

| Music Concepts | Explanation |
| --- | --- |
| Musical Scale | Music is arranged with symbols on a “scale”. We use a scale since the notes are divided by “octaves”. An octave represents eight steps between the equivalent note that is at either double or half the frequency. For example, A4 is 440 hertz (Hz), while A5 is 880 Hz (twice the value of A4), while A3 is 220 Hz (half of A4). |
| Beats per minute/Tempo | Have you ever noticed how some songs are slow while others are fast? Every song has its own “tempo” or speed. The speed is determined by the number of “beats per minute”. A slow song may have 50 beats per minute while a faster song may have 100 beats per minute. |
| Quarter, half, and whole notes | Notes can be shorter or longer depending on the song. One way for a musician to write a song with notes of different durations is to use whole notes, half notes, quarter notes, eighth notes, and sixteenth notes. One whole note equals four quarter notes or two half notes. One half note is equal to two quarter notes and so on. For a song that has 4-4 measures, it means that each measure of the song has 4 beats. A whole note would be equal to 4 beats (hence, one measure). How many half notes could fit into a 4-4 measure? If you said two, you are correct! Since each half note is equal to two beats and there are four total beats per measure, two half notes is equal to four beats. So now you can see how a song with 100 beats per minute will have faster whole notes than a song with 50 beats per minute! Not all whole notes equal the same time duration! |

| Application of notes and beats per minute | Twinkle, Twinkle, Little Star is a song that has multiple notes and uses 4-4 time. Try to code this song with your calculator. |
| --- | --- |

Note - Another useful resource for understanding musical concepts, frequencies and notes can be found

http://www.sengpielaudio.com/calculator-notenames.htm

| Setup Project: |  | Supplies: |
| --- | --- | --- |
| Students may work in groups of two or three. |  | - TI-84 Plus CE Python Calculator - Unit to Unit Cable - TI-Innovator Hub |
| The Student Activity | The Teacher Activity |  |
| Students work in small groups with their calculator and supplies for this activity. They will practice the guidance modeled by your teacher. | Review and introduce the calculator and Hub commands needed for this activity. - Start the Python app - Insert and name a new Python program - Connect Handheld and Innovator Hub |  |

The following “Challenges” will provide a guided approach to building up from basic coding skills to a more complex final program that will enable students to play a song of their choosing. The screen shots below show some of the modules and how to access them.

Fig. 1: Starting a new Program

Fig. 2: Selecting Program Type

Fig. 3: Hub Project with Modules

To start a new program, press [prgm] and then press 2: Python App to start a new Python program (Fig. 1). Since we will be using the TI -Innovator Hub throughout these challenges, click on Types and select 6: Hub Project (Fig. 2). This will automatically import two of the commonly used modules (Fig. 3).

Challenge 1: Start a program called “MS01” (for consistency, we named all the programs “MusicSound##).

- Select Hub Project as the type and then press [Fns...] and arrow over to [Modul] and select 6:ti_hub... Since the speaker is built in to the bottom of the hub, press 1: Hub Built-in devices ...

Fig. 4: Hub Built-in Devices

Fig. 5: Hub Sound Module

Import the sound module, 3: Sound.

- When you press [Fns...] again and arrow over to [Modul], you’ll now find 8: Sound ...

- Set the speaker on the Hub to play a tone of 440 Hz for 2 seconds. Write the program using the sound.tone (freq, time) method (Fig. 7).

Fig. 7: Sample Program

Fig. 6: Hub Sound Output

Explore different tones by changing the frequency measured in hertz, vibrations per second (abbreviated as Hz), and the time.

What is your favorite frequency? Give this tone an appropriate name. For example, sound.tone (100,5) could be named “century.”

Try several different tones. What is the lowest tone you can hear? What is the highest?

Now try this: Add another line with a different frequency, for example sound.tone (263,2). Run the program. What do you notice? Right, only one sound played. The handheld processed the code so fast that the Hub didn’t have a chance to play the first note. To fix this, add a sleep() command between the two sound.tone lines of code. See the sample program below.

PROGRAM: MS01

from ti_hub import *

from ti_system import *

from time import *

import sound

sound.tone(440,2)

sleep(2)

sound.tone(263,2)

Challenge 2:  Write a program called “MS02”. Make a door bell and use the [enter] key on the calculator to ring the chime.

Fig. 9: Sample Program

Fig. 8: Copying and Pasting Code

Most common door bells sound two tones that are a perfect 4th apart. An example is D#5 and G5. You can find a complete list of notes and corresponding frequencies here.

- In the first challenge, we simply pressed [Run] to start the program. This time, we are adding an input command that prompts the user to start the program.

- Press [ Fns ... ], arrow over to [I/O] and select 2: input ().  Pressing the [Alpha] and [+] keys will add quotes to begin the text string. Type something like “Press [enter] to start”.

- Duplicate the lines of code with the frequencies and the sleep () command to make the doorbell ring twice.

- Shortcut (Fig. 8): Place the cursor at the line you want to duplicate, press [Tools] and select 6: Copy Line, move the cursor to the new location, press [Tools] and select 7: Paste Line Below.

PROGRAM: MS02

from ti_hub import *

from ti_system import *

import sound

input('Press [enter] to start')

#”G5"

sound.tone(784,.8)

sleep(.8)

#"D#5"

sound.tone(622.25,.8)

sleep(.8)

sound.tone(784,.8)

sleep(.8)

sound.tone(622.25,.8)

A familiar sound of many doorbells are the Westminster Quarters. Challenge your students to modify the code above to duplicate the sound of those famous bells. The combinations of the four pitches provided by these quarter bells can be found here.

Challenge 3:  Write a program called “MS03” that plays the Major C scale from C4 to C5. Each note should be played for a variable length of time using a variable named ‘dur’ (for duration).

Fig. 11: Input Request

Fig. 10: Type of variable

Start a new program and import the same modules as in MS01 or MS02.

- Now, let’s define the variable ‘dur.’ Press [Fns...] and arrow over to [Type], select 1: int().

- Press [Fns...] again and arrow over to [I/O] and select 2: input()

- Type something like “For how long?” inside the parantheses (see Fig. 12).

Fig. 13: Sample code

Fig. 12: Input Request Statement

Now enter each note from C4 to C5 and play it for the variable length ‘dur.’ Use the “Piano Key Reference Sheet” on the last page for the frequencies.

PROGRAM: MS03

from ti_hub import *

from ti_system import *

import sound

dur=int(input("For how long?"))

"C4"

sound.tone(262,dur)

sleep(dur)

"D4"

sound.tone(294,dur)

sleep(dur)

"E4"

sound.tone(330,dur)

sleep(dur)

"F4"

sound.tone(349,dur)

sleep(dur)

"G4"

sound.tone(392,dur)

sleep(dur)

"A4"

sound.tone(440,dur)

sleep(dur)

"B4"

sound.tone(494,dur)

sleep(dur)

"C5"

sound.tone(524,dur)

sleep(dur)

There is another way to play musical notes on the TI-Innovator Hub. Instead of using the sound.tone(494,dur) command, you can use sound.note(C4,dur) which is also found in the Sound... module. The note names can be UPPERCASE or lowercase (“C4” or “c4”).

Challenge 4:   Create a program called “MS04”. Make a metronome that plays 10 beats with a tone of middle C4 with a  duration of a ¼ note at 100 Beats per minute. To do this you will need to determine the how long a quarter note is (in seconds) based on the information above. A rest of .1 seconds should be used between beats (see Fig. 15, sleep (.1)). Try a double time tempo of 200 Bpm. Also try using a 1/2 or whole note.

Guidance for Challenge 4:

- Loops are used to repeat a set of commands. A “for i in range (size)” loop repeats a specified number of times (see Fig. 14).

Fig. 15: Sample code

Fig. 14: “for i in range (size)” Loop

Students will need to determine the length of time per beat at 100 beats per minute for a quarter note (which is one beat). They can do this by dividing 60 seconds/minute by 100 beats per minute, which equals 0.6 seconds/beat.

PROGRAM: MS04

from ti_hub import *

from ti_system import *

from time import *

import sound

"1/4 note at 100 Bpm"

dur=.6

for i in range(10):

sound.tone(262,dur)

sleep(dur)

sleep(.1)

Fig. 16: Part of sample code MS05

Challenge 5:   Write a program called “MS05”. Use the sheet music for “Twinkle, Twinkle Little Star” to play the first bars of the song on your Hub. An example can be found here. Refer to the “88 Piano Keys” reference sheet found at the end of this document to convert notes to frequencies.

PROGRAM: MS05

from ti_hub import *

from ti_system import *

from time import *

import sound

input( "Press enter to start" )

sound.tone(262,.6)

sleep(.6)

sleep(.1)

sound.tone(262,.6)

sleep(.6)

sleep(.1)

sound.tone(392,.6)

sleep(.6)

sleep(.1)

sound.tone(392,.6)

sleep(.6)

sleep(.1)

sound.tone(440,.6)

sleep(.6)

sleep(.1)

sound.tone(440,.6)

sleep(.6)

sleep(.1)

sound.tone(392,1.2)

sleep(1.2)

sleep(.1)

sound.tone(349,.6)

sleep(.6)

sleep(.1)

sound.tone(349,.6)

sleep(.6)

sleep(.1)

sound.tone(330,.6)

sleep(.6)

sleep(.1)

sound.tone(330,.6)

sleep(.6)

sleep(.1)

sound.tone(294,.6)

sleep(.6)

sleep(.1)

sound.tone(294,.6)

sleep(.6)

sleep(.1)

sound.tone(262,1.2)

sleep(1.2)

sleep(.1)

Challenge 6:  Surely there has to be a way to code music to play on the TI_Innovator Hub without writing each note and its duration in a separate line of code, repeating the same thing over and over?

Well, there is. Let’s write a program called “MS06” and use two lists, L1 and L2, with frequencies and durations and a for ... loop to play the same “Twinkle, Twinkle Little Star” song.

Fig. 18: Lists with Notes and Duration

Fig. 17: Defining Variables for the lists

- Start a new program – MS06 - with the same modules as in MS05.

- The notes/frequencies and duration for each need to be stored in two lists, L1 and L2 (see Fig. 17). This can be done before starting the Python program or now by temporarily quitting out of Python.

PROGRAM: MS06

from ti_hub import *

from ti_system import *

import sound

input("Press enter to start")

notes=recall_list("1")

dur=recall_list("2")

for i in range(len(notes)):

sound.tone(notes[i],dur[i])

sleep(dur[i])

sleep(.1)

Fig. 19: For .... loop

- Back in MS06, press [Fns ...],    [Modul]      4: ti_system...     2: var = recall_list (“name”).

- Define ‘notes’ and ‘dur’ as variables and enter “1” and “2” for the list names (see Fig. 16).

- We are going to use a ‘For .... loop’ to recall the notes and their duration from the lists. Press [Fns ...],    [Ctl]    4: For i in range (size) and enter (len(notes)) for the ‘size’ of the loop. How many times the loop is repeated depends on the number of notes (i) in the list.

- As we did in previous programs, let’s use the sound.tone (freq, time) method (Fig. 19). Instead of entering the actual values for frequency and duration, we have the program recall those values from the lists.

Final Challenge:

- For the final challenge, have the students find the sheet music for a simple song and play it on the Hub.

- Have them use the skills that they have built through the previous challenges and develop a program that can play the song of their choice.

A Little Music Theory

from: 10 Minutes of Code: Python and TI-Innovator™ Technology,

Unit 2: for loops with the TI-Innovator™ Hub, Skill Builder 3

Musical notes are determined by the frequency of a vibrating object such as a speaker, drumhead, or a string in a guitar or piano. The notes of the musical scale have a special mathematical relationship. There are 12 steps in an octave. If a note has frequency f, then the very next note has frequency f × 12√2.

Multiplying a note’s frequency by 12√2 or 2 1/12  (the twelfth root of 2) twelve times results in a doubling of the original frequency, so the last note in the octave (or the first note in the next octave) has frequency of f ×(21/12)12 = 2 × f. For example, if a note has a frequency of 440 Hz, the same note an octave above it has a frequency of 880 Hz, and the same note an octave below it has a frequency of 220 Hz. The human ear tends to hear two notes that are an octave apart as being essentially "the same", due to closely related harmonics. For this reason, notes that are an octave apart are given the same name in the Western system of music notation - the name of the note one octave above C is also C. The 12 intervals between these notes are called ‘semitones’.

In this project we’ll take advantage of the 21/12 principle to generate the 12 notes in an octave (both naturals and sharps) plus the first note in the next octave. Middle C (the lowest note in the staff pictured above) has a frequency of 261.64 Hz. An octave above Middle C, also known as Treble C, has a frequency 2 × 261.64 Hz or 523.28 Hz. There are 12 steps (semitones) between these two notes, and each step is 21/12 times the note before it.

88 - Piano Key Reference Sheet

Use this as a reference for identifying notes that you would like to use in your song based on notes, frequencies or key location.

Frequencies (in hertz…Hz) are listed below the white keys and above the black keys. For example, A0 is equal to 27.5 Hz while A4 is equal to 440 Hz. Notes start from A0 and go to C8 from left to right. Notice that as you move from left to right, each frequency for a note (B for example) roughly doubles. B0 is 30.50 Hz while B1 is 61.74 Hz and B2 is 123.74 Hz and so on.