| Overview of the Project |  |
| --- | --- |
| Students will learn how to use a sensor input (ultrasonic ranger) to control an output (speaker frequency). Students will associate various sound frequencies with the changing distance of an object relative to the ranger. By associating certain frequencies with certain distances, they effectively create a STEM Slide Whistle. They will create the relationship between distance and frequency through a linear equation. | Objectives: |
|  | - Use an input (ranger) to control an output (speaker). - Use a while loop in a program. - Define and use variables in a program. - Print variable values on the screen. - Construct a physical system (hardware) that will interact with software (your code). - Use a linear regression to determine the numeric relationship between distance (input) and sound (output). |
| Technology and Materials Needed (30 students – 10 groups of 3) |  |
| - 3 TI-Nspire CX II Teacher Kits - 10 calculators/1 docking station per kit - 10 TI-Innovator Hubs - 1 per 3 students - 10 Grove – Ultrasonic Distance Sensors - 1 per 3 students - 10 paper towel tubes - 10 scissors - 10 popsicle sticks - Masking tape or hot glue - Hole Puncher - Cardstock or Manila Folder - Circle sheet (end of this document) | 3 x TI-Nspire CX II Teacher Kit: SKU - NSCX2/TPK/2L1 |
|  | TI-Innovator Hub: SKU – STEM/PWB/2L1/E 10 x |
|  | 10 x TI Ultrasonic Ranger Module: SKU-STEMRNG/ENV/9L1 |

| Coding the Ultrasonic Ranger |  |  |  |
| --- | --- | --- | --- |
| - Open the .tns file, “slide_whistleV2.0” and move to page 1.2. - In the space above, “while get_key()!= “esc”:”, add an input device. - Press [Menu] >8 TI Hub >3 Add Input Device >2 Ranger - Enter ‘dist’ as the variable name - Click [tab] to advance to “port”. Choose IN 1. |  |  |  |

| - Move down to “◇◇d=”. Put the cursor to the right of the =. - Press the [var] to choose the variable “dist”. |  |  |
| --- | --- | --- |
| - Once “dist” is pasted, add a decimal point. This will invoke a menu to pop up. Choose “measurement”. |  |  |
| - Move one line below “#print the distance”. - Choose [menu] >4 Built-ins > 6 I/O > 1 print() |  |  |
| - Add “d” inside the ( ). |  |  |
| - Plug the ranger into “IN 1” on your TI-Innovator Hub. - Run the program by pressing [ctrl] and then ‘r’. - Move the ranger around you to see how the distance changes. The rangers range is between 2 to 350 cm (0.02 m to 3.5 m) and has a 1 cm resolution (sensors ability to detect a change). The default unit of measure on the calculator is meters (m). - Stop the program (reading the distance) by pressing "esc" |  |  |  |

| Coding the Sound |  |  |
| --- | --- | --- |
| - Move to page 1.4 (or the page with a python editor with a program called, “slide_whistle2.py”) in your TI-Nspire document, “slide_whistle”. - Insert a sound command below the note that says, “#insert sound command” – [menu]/8 TI Hub/ 2 Hub Built-in Devices / 3 Sound Output/ 1 tone(frequency,time). |  |  |
| - Make sure ‘frequency’ is highlighted. - Then press [var] and choose ‘freq’. - Move to ‘time’. Press [var] again and select ‘time’ from the menu. - Run the program [ctrl] - r |  |  |
| - Use the frequency chart to try some different sounds. For time, enter the value in seconds (ex. 3 would equal 3 seconds that a frequency will play). - To try/sample more frequencies just press ctr-r again to re-run the program. |  |  |

| Build a Slide Whistle |  |  |
| --- | --- | --- |
| - Trace the slot to be cut on the paper towel tube with the popsicle stick. - When creating the slot for the slide, use a hole puncher if possible. This way there is a “pilot hole” for the scissors to easily cut the length of the tube. |  |  |
| - The slot should look similar to the image shown to the right. - Next, use one end of the tube to trace a circle using cardstock or oak tag paper. |  |  |
| - If using printable tag paper sheets, you could opt for the circle sheet (at the end of this document) which prints out 20 circles at once. They are 1.5” wide which is the diameter of a typical paper towel tube. |  | Full sheet at the end of this document |
| - You will tape the circle onto the popsicle stick. This will be the “slide” part of your slide whistle. - Insert the slide into the tube. The circle should fit inside the tube to where the sides are just touching. This will be used to change the distance that the ranger reports in your program. |  |  |
| - Tape the ranger onto the end of the tube. - Make sure the ranger is plugged into IN 1 on the TI-Innovator hub. |  |  |

| Calibrate the Slide Whistle |  |  |
| --- | --- | --- |
| - Move the slide as far away from the ranger as possible. This will be the first value reading – it will correspond to the lowest tone/frequency. - Go back to page 1.2 and run your ranger program again. Pick a consistent value from the list that appears. Enter that value in the first cell on page 1.6. | First value reading – Lowest tone/frequency |  |
| - Add a low frequency to the cell next to it (B1). |  |  |
| - To get the highest tone/frequency, you’ll move the slide as close to the ranger as possible. - Now move back to page 1.2 and run the ranger program again. Pick a consistent value from the list that appears. - Enter that value in the second cell on page 1.6. - Enter a high frequency next to it (B2). | Second value reading – Highest tone/frequency |  |
| - Move to page 1.7 - Click the x-axis to add ‘dist’ (distance) - Click on the y-axis to add ‘freq’ |  |  |
| - Create a linear regression using these two data points by pressing [menu] > 4 Analyze > 6 Regression > 1 Show Linear (mx+b) |  |  |

- Notice the equation appears on the screen.

- Have students copy down their values for m and b here:

m = _________        b = _________

- In our example, m=-3091.79 and b=1030.2.

These values of the slope and y-intercept will be used to convert distance to frequency.

- Proceed to page 1.8 to see the final program.

Notice the program looks similar to your first program. The difference is the addition of “freq=M*d+B” which says the frequency of the sound will be equal to the slope times the distance (x value on the regression) + the y-intercept.

- Before running the program, students will replace the variables ‘M’ and ‘B’ in the equation with the numerical values they recorded above.

- The next line of code, “sound.tone(freq,.1)” will play that value for .1 seconds at a time. As the distance of your slide changes, so will the frequency!

- On p. 2.1 in the .tns file you can find a working version of the slide whistle code, if you wish to show the class what the end result will be.

- Run the program and make some music using code and math!

EXTENSION (for the “more musically inclined”)

A Little Music Theory

The notes of the musical scale have a special mathematical relationship. There are 12 steps in an octave. If a note has frequency F, then the very next note has frequency F × 12√2.

Multiplying a note’s frequency by 12√2 or 21/12 (the twelfth root of 2) twelve times results in doubling the original frequency, so the last note in the octave (or the first note in the next octave) has a frequency of F ×(21/12)12 = 2 × F. For example, if a note has a frequency of 440 Hz, the same note an octave above it has a frequency of 880 Hz, and the same note an octave below it has a frequency of 220 Hz.

A nice introductory activity can be found at https://education.ti.com/en/activities/ti-codes/python/ti-nspire-cx-ii/10-minutes-innovator in Unit 2: for loops with the TI-Innovator™ Hub, Skill Builder 3: Looping with Sound - A Little Music Theory.

Slide Whistle with five Markings

Data Lists with Notes

In the main part of the activity, we ‘ignored’ this exponential relationship between notes. In order to explore that relationship, we will need more than two points of measurement to be able to perform an exponential regression analysis.

- First, let’s add two pages to the document, L&S and D&S. Add those after the page the ‘slide_whistle_final.py’ progran is on.

- The five marks in the picture on the right represent four sections, 0/12th, 3/12th, 6/12th, 9/12th, and 12/12th from the base note (here 440 Hz) to represent the increments from one note to the same note one octave higher. The picture above shows those values entered into the L&S page in column C.

- The actual distace measurements (column D) were taken from right to left, the shortest (3.6 cm) representing the distance to the end of the tube on the left. To match those distances to the frequencies, we could, of course, calculate each value separately and enter it into column B.

A more elegant way, one that also allows you to change the value of that base frequency more easily, is to calculate them based on the value in A1 and store them to column B.    In our example above,

column B =

While still on the L&S page, let’s carry out an exponential regression analysis. Placing the curser somewhere to the right in the spreadsheet (H1), press [ menu ] > 4  Statistics > 1  Stat Calculations >                       A  Exponential Regression ...... in the form, a* b^x.  The values for a, b, and r2, which shows how well the data fit the selected regression model, will be pasted in the column you chose.

- Have students copy down their values for a and b here:

a = _________        b = _________

Sample code for exponential version

- The rest of the activity follows the steps outlined above in the “linear version.”

- In the sample code shown on the right, var=recall_value(“stat.a”) and var=recall_value(“stat.b”) will now recall the values for a and b we obtained in step 4.) above. Those functions are found under                    [ menu ] > A More Modules > 3  TI System > 2 recall_value (“name”).

Using Trace to “predict” Frequencies

By graphing the exponential equation on a Graphs page, you can now also use the trace function, [ menu ] > 5  Trace, to ‘predict’ frequencies for a given length (i.e. distance from the Ranger).

- Again, run the program and make some fun music using Python code and math!