| Function | Example | Behavior |
| --- | --- | --- |
| from module_name import * | from ti_hub import * | Imports all the functions in the ti_hub module for use in the program. The ti_hub module includes all the necessary additions needed for the Path to STEM Projects. |
| var=analog_in(“port”) | dial=analog_in("BB 5") | Controls an input device through port BB5 on the breadboard. In the first example, it associates port BB5 with the name, “dial” (a potentiometer is attached to the breadboard). To find this in the menu, go to [menu]/8. TI Hub/3. Add Input Device/8. analog In |
| for index in range(stop value): block | for n in range(10): | Repeats the statements in the block 10 times, If n is less than the stop value, 10, the loop continues to repeat. The block starts with a colon and includes the indented lines that follow. |
| var.range(min,max) | bright.range(0,255) | The range option allows a user to set a range of numeric values for a sensor or device between a user defined minimum value and a maximum value. Usually this is used to scale the readings of a sensor. To access it, type a decimal after a variable and range will be an option in a popup menu. |
| var.measurement | bright.measurement | The measurement option allows a user to get a numeric reading from a sensor or device. Usually this is from a sensor. To access it, type a decimal after a variable and measurement will be an option in a popup menu. |
| text_at(vertical position,”text”,”horizontal position”) | text_at(6,”RED”,”left”) | text_at is similar to “print” but enables a user to specify the row that the text should display on the calculator screen (1 through 8), and the horizontal position of the message (right, center, left). |
| var=float(input(“text”)) | m=float(input(“m = ?”)) | To generate a prompt in your program, an input is needed (found under “4. Built-ins”, “6. I/O”). If your input requires a number, like the example, choose from either int (integer) or float (decimal) found under “4. Built-ins”, “5. Type”. Define this line as a variable (in the example, it is “m” for slope) so it can be used later. |
| while var(Boolean)var: block | while t<p: heater.set(100) x=thermometer.measurement() t=(m*x)+b text_at(6,"Heating...,Temp= "+str(t),"left") | while loops repeat the statements in the block if the condition at the top of the loop is true. In the example, looping continues until the variable “t” is greater than “p”. |
| if <Boolean expression>: block | if a<s: light.on() | Checks to determine if the value of variable a is less than the variable s. If the statement is “true” then the commands in the if block are executed. Otherwise, the block is skipped. In the example, when a is less than s, then the light is turned on. Otherwise, the program moves to “else” which ensures the light is off. |

See TI-Innovator Hub Technology eGuide for more background on Hub-specific commands – Link

See TI-Nspire CXII Python Programming eGuide for more background on Python commands - Link